LLM On-Device Inference

Benchmark Experiment Report

Experiments E1–E6  |  Device 1 & Device 2  |  March 25–26, 2026

# 1. Executive Summary

This report summarizes results from six benchmark experiments evaluating multiple large language models (LLMs) running locally on two edge devices. The study assessed baseline performance, classification accuracy, agent-chain overhead, cross-device scalability, memory consumption, and cold/warm-start latency.

Key Findings:

• mistral:7b achieved the highest classification accuracy (93%) but consumed the most memory (5.4 GB peak) with load times exceeding 47 seconds.

• phi3:mini offered the best accuracy-to-efficiency balance: 89% accuracy, moderate RAM usage, and reasonable load times.

• Quantized variants (q5, q8) of all models produced empty outputs in inference tasks on Device 2, suggesting a runtime incompatibility.

• qwen2.5:3b and mistral:7b triggered swap memory on Device 1, indicating RAM pressure for larger models.

• Warm-start conditions improved TTFT by 5–15x compared to cold-start across all models.

• tinyllama delivered the fastest token throughput (16+ tok/s) with minimal memory overhead (~680 MB delta).

# 2. Experiment Overview

Six experiments were conducted across two devices. Device 1 ran all six experiments; Device 2 served as a secondary environment for experiments E1–E3, E5, and E6 (comparison baseline). The full-precision models were tested on Device 1, while quantized variants were tested on both devices.

Models tested: tinyllama, phi3:mini, qwen2.5:3b, mistral:7b, and their quantized variants (q5, q8).

Experiments ran across 20–30 iterations per condition unless otherwise noted.

# 3. E1 — Baseline Performance

Objective: Establish baseline inference speed, TTFT, and resource usage for each model generating 100 tokens across 20 warm runs (run 1 excluded as cold-start).

## 3.1 Results — Device 1

| Model | Load Time | Avg Tok/s | Avg TTFT (ms) | Peak RAM | Avg Total Time |
| --- | --- | --- | --- | --- | --- |
| tinyllama | 1.81 s | 16.35 | 167.1 | 1,306 MB | 6.32 s |
| phi3:mini | 12.76 s | 4.57 | 275.2 | 4,258 MB | 22.18 s |
| qwen2.5:3b | 12.98 s | 5.64 | 460.4 | 2,748 MB | 13.43 s |
| mistral:7b | 17.09 s | 3.49 | 344.8 | 5,375 MB | 28.13 s |

tinyllama was by far the fastest model at 16.35 tok/s with only 1.81 s load time, making it ideal for resource-constrained or latency-sensitive scenarios. phi3:mini and qwen2.5:3b offered a middle ground. mistral:7b generated the slowest output (3.49 tok/s) and required the longest load time at 17.09 s.

Note: Device 2 results for E1 showed all models producing 0 tokens, consistent with the pattern seen across experiments on Device 2 where model inference did not complete — likely due to a runtime environment issue.

# 4. E2 — Classification Accuracy

Objective: Evaluate each model's ability to classify 100 email samples into 5 categories (inquiry, complaint, feedback, spam, urgent). Accuracy is measured as the proportion of correct label predictions.

| Model | Accuracy | Correct / Total |
| --- | --- | --- |
| mistral:7b | 93.0% | 93/100 |
| phi3:mini | 89.0% | 89/100 |
| qwen2.5:3b | 80.0% | 80/100 |
| tinyllama | 27.0% | 27/100 |
| tinyllama-q5/q8 | 0.0% | 0/100 (no output) |
| phi3-q5/q8 | 0.0% | 0/100 (no output) |
| qwen-q5/q8 | 0.0% | 0/100 (no output) |
| mistral-q5/q8 | 0.0% | 0/100 (no output) |

mistral:7b and phi3:mini are the clear leaders in accuracy. qwen2.5:3b reaches 80%, which may be sufficient for many production pipelines. tinyllama's 27% accuracy reflects its smaller parameter count and limited instruction-following capability. All quantized variants returned empty outputs on Device 1, scoring 0% — the quantized versions appear to lack the instruction-tuning required for this classification task format.

# 5. E3 — Agent Chain Overhead

Objective: Measure the overhead introduced by chaining three sequential inference steps (agent pipeline) compared to a single equivalent inference call. An overhead ratio < 1.0 indicates the chain ran faster than 3× single time (parallelism benefit); > 1.0 indicates overhead. All 30 runs per model completed successfully on Device 1.

| Model | Avg Overhead Ratio | Chain Completion | Notes |
| --- | --- | --- | --- |
| tinyllama | 0.884 | 30/30 | Below linear (efficient pipelining) |
| phi3:mini | 1.035 | 30/30 | Near-linear, slight overhead |
| qwen2.5:3b | 0.834 | 30/30 | Best ratio — fastest chaining |
| mistral:7b | 1.137 | 30/30 | Noticeable overhead per step |

qwen2.5:3b exhibited the best chaining efficiency (0.834 ratio), suggesting effective context reuse across steps. tinyllama was similarly efficient at 0.884. phi3:mini ran near-linear (1.035), and mistral:7b showed the most coordination overhead (1.137). All models on Device 2 returned failed chain completions (0 tokens), consistent with the runtime issue observed there.

# 6. E4 — Cross-Device Scaling (Device 1)

Objective: Evaluate how model performance changes with prompt length (short: 25 chars, medium: 177 chars, long: 387 chars) on Device 1. Results below are from warm runs (run 1 excluded).

## 6.1 phi3:mini Performance by Prompt Length

| Prompt Type | Avg Tok/s | Avg TTFT (ms) | Notes |
| --- | --- | --- | --- |
| Short (25c) | 3.96 | 351.1 | Baseline |
| Medium (177c) | 3.87 | 324.2 | Slight TTFT improvement |
| Long (387c) | 5.81 | 200.4 | Higher throughput — longer context |

## 6.2 mistral:7b Performance by Prompt Length

| Prompt Type | Avg Tok/s | Avg TTFT (ms) | Notes |
| --- | --- | --- | --- |
| Short (25c) | 3.93 | 272.5 | Fastest TTFT |
| Medium (177c) | 3.88 | 280.3 | Stable throughput |
| Long (387c) | 3.83 | 280.4 | Minor degradation |

phi3:mini shows throughput gains with longer prompts, likely due to reduced startup overhead relative to generation time. mistral:7b remains very consistent across prompt sizes, indicating stable throughput regardless of context length.

# 7. E5 — Memory Profiling (Device 1)

Objective: Profile RAM consumption across model loading, inference, and unloading phases. Key metrics include model RAM delta (memory increase after loading), peak inference RAM, and whether swap memory was activated.

| Model | RAM Delta | Peak RAM | RAM Util% | Load Time | Swap Used |
| --- | --- | --- | --- | --- | --- |
| tinyllama | 679.9 MB | 1,360 MB | 17.5% | 6.76 s | False |
| tinyllama-q5 | ~0 MB | 676 MB | 8.7% | <0.01 s | False |
| tinyllama-q8 | ~0 MB | 678 MB | 8.7% | <0.01 s | False |
| phi3:mini | 3,629 MB | 4,306 MB | 55.3% | 18.75 s | False |
| phi3-q5/q8 | ~0 MB | 687 MB | 8.8% | <0.01 s | False |
| qwen2.5:3b | 2,041 MB | 2,735 MB | 35.1% | 5.64 s | True (swap) |
| qwen-q5/q8 | ~0 MB | 732 MB | 9.4% | <0.01 s | False |
| mistral:7b | 4,673 MB | 5,407 MB | 69.5% | 47.91 s | True (swap) |
| mistral-q5/q8 | ~0 MB | 703 MB | 9.0% | <0.01 s | False |

Full-precision models consume dramatically more RAM than their quantized counterparts. mistral:7b used 69.5% of system RAM (7,785 MB total) and triggered swap, indicating it is at the edge of Device 1's capacity. qwen2.5:3b also triggered swap. Quantized variants (q5, q8) showed near-zero RAM delta — these models appear to have failed to load properly, since inference times were sub-millisecond and output was empty.

Device 2 showed all models at ~19% RAM utilization with negligible deltas, confirming that none of the models actually loaded for inference in that environment.

# 8. E6 — Cold vs Warm Start Latency

Objective: Compare first-token latency (TTFT) and throughput under cold-start (model loaded fresh) versus warm-start (model already cached in memory) conditions. Each model ran 5 cold and 20 warm iterations.

| Model | Condition | Avg Tok/s | Avg TTFT (ms) |
| --- | --- | --- | --- |
| tinyllama | Cold | 9.21 | 2,505 |
| tinyllama | Warm | 12.89 | 349 |
| phi3:mini | Cold | 3.58 | 6,204 |
| phi3:mini | Warm | 5.07 | 522 |
| qwen2.5:3b | Cold | 4.49 | 5,674 |
| qwen2.5:3b | Warm | 6.60 | 644 |
| mistral:7b | Cold | 2.26 | 11,395 |
| mistral:7b | Warm | 3.23 | 763 |

Warm-start conditions reduce TTFT dramatically across all models. tinyllama benefits most proportionally (7.2× TTFT reduction), while larger models like mistral:7b reduce TTFT from ~11 seconds to under 1 second. Throughput also improves under warm conditions as the model avoids re-initializing weights from disk. These results strongly suggest caching strategies (model-pinning) would substantially improve real-world deployment latency for larger models.

# 9. Conclusions & Recommendations

## 9.1 Model Selection by Use Case

High-accuracy NLP tasks (classification, instruction-following): Use mistral:7b or phi3:mini. Both exceeded 89% on email classification with full reasoning capability.

Low-latency, high-throughput tasks: Use tinyllama. At 16+ tok/s with sub-2s total inference time, it is optimal for speed-critical applications where accuracy requirements are modest.

Agent/chaining pipelines: qwen2.5:3b offers the best overhead ratio (0.834) and reasonable accuracy (80%), making it a balanced choice for multi-step workflows.

## 9.2 Device 2 Findings

All models on Device 2 consistently produced zero tokens across every experiment. Memory profiles showed no significant RAM delta after model loading. This is strongly indicative of a runtime environment configuration issue (e.g., incompatible Ollama build, missing GGUF backend, or permission restrictions) rather than hardware limitations. Device 2 should be investigated and reconfigured before further experiments.

## 9.3 Quantization Findings

All quantized models (q5, q8 variants) on Device 1 produced empty outputs during inference. This suggests the quantized GGUF files may not be instruction-tuned, or the inference prompt format is incompatible with the quantized model configuration. Testing with proper quantized-instruction variants is recommended before ruling out quantization as a deployment strategy.

## 9.4 Deployment Recommendations

1. Implement model-pinning/caching for production deployments to benefit from warm-start gains.

2. Avoid mistral:7b on devices with <8 GB RAM due to swap activation risk.

3. Re-test quantized variants with properly instruction-tuned quantized models.

4. Investigate and resolve the Device 2 runtime environment before proceeding with cross-device experiments.

5. Consider phi3:mini as the primary production model offering the best accuracy-memory-speed tradeoff.

# 10. Appendix — File Reference

The following 11 CSV files were analyzed in this report:

• e1_baseline_device1 / device2 — Baseline performance across 4 models × 20 runs

• e2_accuracy_device1 / device2 — 1,200-row classification accuracy datasets

• e3_overhead_device1 / device2 — Agent chain overhead (120 rows each)

• e4_crossdevice_device1 — 180 rows cross-device prompt-length scaling

• e5_memory_device1 / device2 — Memory profiling (12 rows each)

• e6_coldwarm_device1 / device2 — Cold/warm latency (100 rows each)

Total records analyzed: ~2,900+ rows across all experiments.

Report generated: April 3, 2026